ZOVUYO INNOVATIONS

Smart GrowKit

Project Status Report & Handover Summary

Prepared by: Oluhle Jawe, Founder

Incubatee — Innovate Durban

Durban, KwaZulu-Natal, South Africa

Date: 30 August 2026

# Contents

# 1. Executive Summary

The Smart GrowKit is a compact, IoT-controlled hydroponic growing system designed for urban South African households. It combines an ESP32 microcontroller, a five-sensor environmental monitoring array, relay-controlled actuators, and a cloud monitoring stack to automate the growing of leafy greens in balcony- and kitchen-scale spaces.

The project has completed its full design and specification phase. Hardware architecture, power design, PCB and enclosure planning, requirements specification, operational scenarios, and system assumptions are all documented and stable. Physical components have been procured, with three items outstanding.

Work has now crossed into the firmware development phase. The development toolchain is installed and verified, and the first firmware build compiled successfully. The immediate blocking item is the USB driver installation required to flash the physical ESP32 board — everything downstream of that point is queued and unblocked once the board enumerates.

| Phase | Area | Status |
| --- | --- | --- |
| 1 | Requirements & specification | Complete |
| 2 | Hardware & power design | Complete |
| 3 | Component procurement | Substantially complete — 3 items outstanding |
| 4 | Firmware development | In progress — toolchain verified, first build successful |
| 5 | Cloud stack & integration | Not started — planned |

# 2. What Has Been Completed

## 2.1 Design & Specification Documentation

Five formal documents form the design baseline for the system. All are included in the accompanying archive:

- System Requirements Specification — 20 functional requirements (monitoring, control, alerting, data) and 22 non-functional requirements spanning reliability, response time, safety, maintainability, power efficiency, usability, environmental limits, and cost, each with a stated verification method.

- Requirements Analysis — actor identification and eight full use cases (UC-1 through UC-8), covering environmental monitoring, environmental control, nutrient delivery, alerting, status reporting, plant profile configuration, maintenance, and portable setup.

- Design Summary — power design decisions, PCB and protoboard layout planning, 3D control box mechanical plan, and the three-tier vertical form factor intended for urban living.

- Operational Scenarios — day-by-day operating logic for vegetative growth, bolting prevention and maturation, harvest, and system reset, including the full parameter matrix and control logic rules.

- System Assumptions — indoor environmental assumptions, Eskom power and load-shedding assumptions, Wi-Fi connectivity projections, South Africa-specific economic and regulatory factors, and an assumption tracking matrix with confidence levels.

## 2.2 Hardware Design Decisions Locked In

The following engineering decisions are settled and should not be revisited without cause:

| Area | Decision | Rationale |
| --- | --- | --- |
| Power supply | 12V 8A | Peak load calculated at 5.7A; 8A keeps the unit at ~70% capacity, preventing overheating and extending lifespan |
| Protection | 10A fast-blow ceramic fuse | Covers 5.7A load plus LED inrush current; ceramic avoids the arcing risk of glass under high-current failure |
| 12V → 5V | LM2596 or MP1584 buck converter | Powers sensor modules and logic level shifters |
| 5V → 3.3V | AMS1117 regulator | Cleaner signal than stepping directly off the motor-noisy 12V rail |
| Grounding | Star-grounding from PSU | Daisy-chaining the 3A heater ground to the ESP32 causes ground bounce and brown-out crashes |
| Heavy wiring | 18 AWG | 3A heater and 2A LED runs — prevents voltage drop at the far end of the cable |
| Signal wiring | 22 AWG | Sensors, ESP32 power, PWM signal lines |
| Main connector | XT60 / XT30 | High-current RC-grade; will not melt like a standard barrel jack |
| Sensor connectors | GX12 aviation (3–4 pin) | Waterproof with a threaded lock so cables cannot pull out of the enclosure |

Layout is organised into three isolated physical zones: a high-power zone at the DC input edge, an analog zone (pH/CO₂) placed as far as possible from the PWM switching to avoid crosstalk, and the logic zone. The enclosure design uses a 35mm DIN rail for clip-in relay modules, a 40mm intake fan over the ESP32 and buck regulators, weep holes or a breathable vent plug for exhaust, and bottom-mounted PG7/PG9 cable glands so condensation drips off before entering the box.

## 2.3 Business & Incubator Milestones

- A Business Progress Report was prepared and submitted to Innovate Durban, covering executive summary, hardware design, firmware development, cloud architecture, operational scenarios, risk assumptions, project schedule, and budget.

- A companion submission email was drafted and sent, expressing gratitude for Innovate Durban's support and patience and noting that design videos would follow.

- Design visuals are hosted externally rather than embedded in the source documents. Any future document that needs images should have screenshots uploaded directly.

## 2.4 Firmware Environment

- VS Code installed with the PlatformIO IDE extension.

- Project created targeting the ESP32 Dev Module board using the Arduino framework.

- A blink and serial "hello world" sketch written in main.cpp.

- First build completed successfully — RAM usage 6.6%, Flash usage 20.5%. Substantial headroom for the full control firmware.

- Confirmed: LED_BUILTIN is not reliably defined on the ESP32 Dev Module. Use #define LED_PIN 2 — GPIO 2 is the onboard blue LED on most variants.

# 3. Exactly Where Work Stopped

The last active task was flashing the first firmware build to physical hardware. The build succeeded, but the ESP32 was not connected at the time, so the upload step was never executed.

| # | Step | State |
| --- | --- | --- |
| 1 | Install VS Code + PlatformIO extension | Done |
| 2 | Create ESP32 Dev Module / Arduino framework project | Done |
| 3 | Write blink + serial sketch using GPIO 2 | Done |
| 4 | Compile (Build) | Done — succeeded |
| 5 | Install USB-to-serial driver (CP2102 or CH340) | NOT DONE — current blocker |
| 6 | Connect ESP32 and confirm COM port appears | Pending step 5 |
| 7 | Upload firmware to board | Pending step 6 |
| 8 | Open Serial Monitor at 115200 baud and confirm output | Pending step 7 |

To resume: identify the USB-to-serial chip on the ESP32 board (the small square IC near the USB port — usually marked CP2102 or CH340), install the matching driver, then plug the board in and check that a new COM port appears in Device Manager. From there, PlatformIO's Upload button completes step 7, and the Serial Monitor at 115200 baud confirms the firmware is running.

# 4. Outstanding Items

## 4.1 Missing Components

Three parts were identified as missing during the procurement review and are required before the power stage can be safely assembled:

| Component | Purpose | Priority |
| --- | --- | --- |
| 10A ceramic fast-blow fuse | Master circuit protection on the 12V input | Critical — do not power the full build without it |
| 1N4007 flyback diodes | Suppress inductive kickback from pump, fan, and relay coils | Critical — protects the ESP32 and relay drivers |
| IRFZ44N MOSFET | Switching for the high-current heater / LED load | High |

## 4.2 Cloud Stack (Not Yet Started)

The planned stack is entirely free-tier and self-hostable:

- HiveMQ — MQTT broker for device-to-cloud messaging

- Node-RED — flow-based logic, routing, and rule engine

- InfluxDB — time-series storage for sensor history

- Grafana — dashboards and historical trend visualisation

- Telegram Bot — push alerting channel

# 5. Recommended Next Steps

| Order | Task | Notes |
| --- | --- | --- |
| 1 | Install CP2102 / CH340 USB driver | Unblocks all firmware work; 15 minutes |
| 2 | Upload blink sketch and verify on Serial Monitor | Confirms the full toolchain end to end |
| 3 | Order the three missing components | Fuse, flyback diodes, MOSFET — lead time is the risk, order early |
| 4 | Bring up sensors one at a time | Temperature → humidity → water level → pH → CO₂. Verify each in isolation before combining |
| 5 | Bench-test relays with a dummy load | Never test switching against the real heater first |
| 6 | Write the control loops | Five loops: lighting schedule, pump cycle, temperature, humidity/fan, pH correction |
| 7 | Stand up MQTT and publish sensor data | HiveMQ first, then Node-RED subscription |
| 8 | Add InfluxDB storage and Grafana dashboard | Historical trends and the demo-facing view |
| 9 | Add Telegram alerting | Maps directly to the alert conditions in the Requirements Analysis |

Steps 1 and 2 are short and should be completed in the next available evening block. Step 3 should be initiated in parallel, since delivery time is the only element outside your control.

# 6. Key Reference Values

Pulled together here so they are in one place when firmware work resumes.

| Parameter | Target / Value |
| --- | --- |
| Serial monitor baud rate | 115200 |
| Onboard LED pin | GPIO 2 |
| Water / air temperature | 18–22 °C (day 20–24 °C, night 18–20 °C) |
| Humidity | 60–70% RH vegetative; 40–60% general leafy greens |
| pH | 5.5–6.5 (target 5.8 ± 0.2) |
| EC | 1.2–2.2 mS/cm depending on growth stage |
| CO₂ | 400–800 ppm |
| Photoperiod | 16 hours on / 8 hours off |
| Light intensity | 250–500 µmol/m²/s |
| Pump cycle | 15 min on / 45 min off |
| Sensor poll interval | 5 minutes (pH 15 min, CO₂ 30 min, EC 1 hour) |
| Cloud upload interval | 15 minutes |
| Peak current draw | 5.7 A at 12 V |
| Reservoir low-level alert | Below 20% capacity |
| Time to harvest | 28–35 days from transplant |

# 7. Archive Contents

The accompanying ZIP archive contains the following:

| File | Description |
| --- | --- |
| GrowKit_Status_Report.docx | This report |
| Design_Summary.docx | Power design, PCB planning, enclosure and form factor |
| Operational_Scenarios.docx | Growth, bolting prevention, harvest and reset operating logic |
| Requirements_Analysis.docx | Actors and use cases UC-1 to UC-8 |
| SYSTEM_ASSUMPTIONS.docx | Environmental, power, connectivity and market assumptions |
| System_Requirements_Specification.docx | Functional and non-functional requirements with verification methods |

Zovuyo Innovations · Smart GrowKit · Status as at 30 August 2026