Requirements Analysis & System Design

Complete Use Case Diagrams and Analysis

System Overview

IoT-powered hydroponic system for growing leafy greens in South Africa, featuring automated nutrient delivery, environmental control, and remote monitoring via ESP32.

Actor Identification

| Primary Actors | Secondary Actors |
| --- | --- |
| End User - Home grower using the system | Cloud Service - External IoT platform for data storage/processing |
| Mobile App User - User accessing system remotely | Alert System - Sends notifications |
| System Administrator - Advanced user/developer (you) | Plant Database - Stores plant-specific requirements |
|  | Calendar/Time Service - Provides scheduling functions |

UC-1: Monitor Environmental Conditions

| Primary Actors | Secondary Actors | Preconditions | Main Flow | Alternate Flow | Post Conditions |
| --- | --- | --- | --- | --- | --- |
| System (automated) | End User | System powered on, sensors calibrated | System continuously polls all sensors at 5-minute intervals | System detects sensor reading outside expected range | Environmental data stored and available for analysis |
|  |  |  | pH sensor measures nutrient solution acidity (target: 5.5-6.5 for leafy greens) | Marks sensor as "needs calibration" |  |
|  |  |  | Temperature sensor monitors water/reservoir temperature (target: 18-22°C) | Sends alert to user |  |
|  |  |  | Humidity sensor tracks ambient humidity (target: 40-60% for lettuce/spinach) | Continues operation with remaining sensors |  |
|  |  |  | CO2 sensor monitors CO2 levels (target: 400-800 ppm) |  |  |
|  |  |  | Water level sensor checks nutrient reservoir level |  |  |

-

UC-2: Control Environmental Conditions

| Primary Actors | Secondary Actors | Preconditions | Main Flow | Alternate Flow | Post Conditions |
| --- | --- | --- | --- | --- | --- |
| System (automated) | End User | Environmental parameter deviates from setpoint | System compares sensor readings with plant-specific targets | User remotely adjusts temperature setpoint | Environment maintained within target ranges |
|  |  |  | If temperature < 18°C), Activate nichrome wire heater via relay | System acknowledges override |  |
|  |  |  | If temperature > 22°C: Activate axial fan for cooling | Operates according to manual settings |  |
|  |  |  | If temperature > 22°C: Activate axial fan for cooling |  |  |
|  |  |  | If humidity < 40%: Reduce fan speed |  |  |
|  |  |  | If CO2 < 400 ppm: Note for future CO2 enrichment capability |  |  |

UC-3: Manage Nutrient Delivery System

| Row | Actor | Trigger / Condition | Steps / Logic |
| --- | --- | --- | --- |
| 1 | System | Scheduled cycle (every 30 minutes) | 1. Activate water pump for 5 minutes. 2. Circulate nutrient solution through root zone. 3. Monitor pH during circulation. 4. Log circulation events. |
| 2 | System | Detects pH outside optimal range (5.5 - 6.5) | 1. Calculate correction amount based on current pH, reservoir volume, and target pH. 2. Activate micro-dosing pump (future enhancement). |
| 3 | System | During pH Correction | 3. Add pH Up/Down solution in small increments. 4. Wait 15 minutes, then remeasure pH. 5. Repeat steps until pH is normalized. |
| 4 | System | Scheduled monitoring & estimation | 1. Estimate nutrient depletion based on: days since last change, plant growth stage, and water level changes. |
| 5 | System | Meets alert condition | 2. Alert user when: reservoir is <20% full, 7 days since last nutrient change, or EC levels are out of range (if sensor available). |
| 6 | End User | Manual intervention / adjustment | Alternate Flow: 1. User adds nutrients manually. 2. User logs amount added via app. 3. System recalculates depletion timeline. Postcondition: Plants receive adequate nutrients at proper pH. |

UC-4: Alert User of Issues

Alert Conditions & Categories

| Alert Level | Condition / Trigger | Examples |
| --- | --- | --- |
| Primary Actor | System |  |
| Secondary Actor | End User, Alert System |  |
| Critical Alerts | Immediate notification required. | • Water pump failure • Power outage detected • Temperature >30°C or <10°C • Water leak detected (via moisture sensor) |
| Warning Alerts | Hourly notification if unresolved. | • pH out of range for >2 hours • Nutrient reservoir <20% full • Any sensor malfunction • WiFi connection lost |
| Informational Alerts | Daily summary notification. | • Daily growth report • Nutrient depletion estimate • Water consumption report • System health status |

Notification Methods

| Notification Method | Description / Use Case |
| --- | --- |
| Mobile App Push Notification | Primary method for all alert levels. |
| SMS | Used for critical alerts, especially relevant for users in South Africa. |
| Email Digest | Used for informational alerts and summaries. |
| Local Buzzer / Indicator | Provides immediate, on-site attention for critical hardware issues. |

UC-5: Report System Status

| Row | Function / Flow | Actor / Trigger | Steps & Details |
| --- | --- | --- | --- |
| 1 | A. Real-time Dashboard | Primary: Mobile App User / End User Secondary: Cloud Service Trigger: User opens app | 1. User opens mobile app or web interface. 2. System displays: • Current sensor readings (color-coded) • System status indicators • Camera feed (future) • Manual override controls • Growth progress photos |
| 2 | B. Historical Reports | Trigger: User requests data | 1. User requests historical data. 2. System provides: • 24-hour parameter graphs • 7-day trend analysis • Nutrient consumption charts • Alert history log |
| 3 | C. Vacation Mode | Trigger: User enables mode | 1. User enables vacation mode. 2. System adjusts to: • Increase system check frequency • Send daily status summaries • Enable extended reservoir capacity logic • Prioritize critical alerts |

UC-6: Configure Plant-Specific Parameters

| Row | Flow / Step | Actor / Condition | Action & Details |
| --- | --- | --- | --- |
| 1 | Primary Actor & Trigger | Primary Actor: End User Secondary Actor: Plant Database Trigger: User begins setup or reconfiguration | (This row sets the context for the use case) |
| 2 | Main Flow: 1. Plant Selection | User selects a plant type from a predefined list. | Options include: • Lettuce: Temp 18-22°C, pH 5.5-6.0, Light 14-16hrs • Spinach: Temp 15-20°C, pH 6.0-6.5, Light 12-14hrs • Kale: Temp 10-25°C, pH 6.0-7.0, Light 14-16hrs • Herbs (e.g., Basil): Temp 20-25°C, pH 5.5-6.5, Light 16-18hrs |
| 3 | Main Flow: 2. Apply Parameters | System receives the plant type selection. | System automatically applies the optimal parameters for the chosen plant, including: • Temperature range • pH range • Light schedule (for LED control) • Humidity preferences • Nutrient schedule |
| 4 | Main Flow: 3. Customization | User reviews the applied settings. | The user has the option to customize any parameter (e.g., temperature, light hours) to override the system's default optimal settings. |

UC-7: Perform System Maintenance

| Row | Maintenance Type / Step | Actor / Condition | Actions & Details |
| --- | --- | --- | --- |
| 1 | Primary Actor & Preconditions | Primary Actor: End User Preconditions: System alert OR scheduled maintenance trigger | (Context for the maintenance activity) |
| 2 | A. Weekly Maintenance | End User performing routine check | 1. Check and calibrate pH sensor. 2. Clean pump filter. 3. Inspect system for algae growth. 4. Verify all tubing and electrical connections. |
| 3 | B. Bi-weekly Maintenance | End User performing comprehensive refresh | 1. Perform complete nutrient reservoir change. 2. Clean entire system with a mild bleach solution. 3. Inspect and prune plant roots if necessary. 4. Replace any worn components (e.g., filters, tubing). |
| 4 | C1. pH Sensor Calibration | End User ensuring sensor accuracy | Procedure: 1. Prepare pH 4.0 and 7.0 buffer solutions. 2. Follow the sensor manufacturer's specific calibration process. 3. Verify the calibration by testing with the second buffer solution. |
| 5 | C2. Temperature Calibration | End User verifying temperature readings | Procedure: 1. Compare the system's temperature reading with a known, calibrated thermometer. 2. Adjust the sensor's offset in the system settings if a discrepancy is found. |

UC-8: Portable Setup & Mobility

| Row | Process Phase | Primary Actor / Context | Steps & Details |
| --- | --- | --- | --- |
| 1 | Context | Primary Actor: End User | (This uses case covers the initial setup and physical relocation of the system.) |
| 2 | Initial Setup | User is preparing the system for first use. | 1. Unfold/carry unit to location (e.g., balcony/kitchen). Max dimensions: 60×40×40cm. 2.Connect the power cable. 3. Fill the reservoir with water. 4. Add nutrient concentrate. 5. Power the system on. 6. Connect to Wi-Fi via the smartphone app. |
| 3 | Relocation | User needs to move the system to a new location. | 1. Drain the reservoir to its minimum level. 2. Power off the system. 3. Carry to new location. Weight when empty: <15kg. 4. Reconnect power. 5. Top up the reservoir with water/nutrients. 6. System resumes normal operation automatically. |

System Component Interactions

| SENSORS | ACTUATORS | COMMS | STORANGE | POWER |
| --- | --- | --- | --- | --- |
| pH | LED Strips | Wi-Fi | Local Log | 12v Input |
| Temperature | Water Pump | Bluetooth | Cloud Sync | DC-DC Conv |
| Humidity | Fan | MQTT | OTA Updates | Backup Battery |
| CO2 | Heater | HTTP API |  |  |
| Water Level |  |  |  |  |