System Requirements Specification

Smart Hydroponic Grow Kit

Functional Requirements

| Category | Requirement ID | Requirement Description | Specification | Verification Method |
| --- | --- | --- | --- | --- |
| Monitor | FR-MON-01 | Temperature Monitoring | Range: 10-40°C ±0.5°C Update interval: 5 minutes Sensor: Thermocouple/DS18B20 | Calibrated thermometer comparison |
|  | FR-MON-02 | Humidity Monitoring | Range: 20-90% RH ±3% Update interval: 5 minutes | Salt calibration test |
|  | FR-MON-03 | pH Level Monitoring | Range: 4.0-9.0 pH ±0.1 Update interval: 15 minutes | Buffer solution calibration (4.0, 7.0, 10.0) |
|  | FR-MON-04 | CO₂ Concentration Monitoring | Range: 400-2000 ppm ±50 ppm Update interval: 30 minutes | Calibration with reference gas |
|  | FR-MON-05 | Water Level Monitoring | Reservoir capacity: 30L ±0.5L Low threshold: 20% capacity | Manual measurement verification |
|  | FR-MON-06 | Nutrient Concentration (EC) | Range: 0-5.0 mS/cm ±0.1 mS/cm Update interval: 1 hour | Calibration solution verification |
| Control | FR-CTL-01 | LED Lighting Control | PWM control: 0-100% ±5% Schedule: 16/8 photoperiod Intensity: 0-500 µmol/m²/s | Lux meter measurement, schedule verification |
|  | FR-CTL-02 | Water Pump Control | Flow rate: 600L/h ±10% Schedule: 15min ON/45min OFF Manual override capability | Flow meter measurement, timer verification |
|  | FR-CTL-03 | Fan Speed Control | PWM control: 25-100% speed Auto mode: Temp/Humidity based Manual control available | Anemometer measurement, response testing |
|  | FR-CTL-04 | Temperature Regulation | Heating: Nichrome wire 0-100W Cooling: Fan speed modulation Setpoint maintenance: ±1.5°C | Temperature stability test over 24h |
|  | FR-CTL-05 | pH Auto-correction | Dosing pumps: 0.1ml increments Adjustment range: 5.5-6.5 pH Delay between adjustments: 15min | pH drift correction test |
| Alerting | FR-ALT-01 | Threshold Breach Alerts | Temperature: <15°C or >30°C pH: <5.5 or >6.5 Water level: <20% CO₂: <400 ppm or >1500 ppm | Simulated breach conditions |
|  | FR-ALT-02 | Component Failure Alerts | Pump failure detection Sensor disconnection Communication failure Power interruption | Component disconnection testing |
|  | FR-ALT-03 | Alert Delivery Methods | Mobile app notification SMS (South African networks) Email Local audible/visual alarm | Multi-channel delivery test |
|  | FR-ALT-04 | Maintenance Reminders | Filter replacement: 30 days pH calibration: 14 days Nutrient change: 21 days System cleaning: 60 days | Schedule-based trigger testing |
| Data | FR-DAT-01 | Local Data Logging | Storage: SD card 8GB minimum Retention: 30 days minimum Format: CSV with timestamp Parameters: All sensor readings | Data retrieval and integrity check |
|  | FR-DAT-02 | Cloud Data Upload | Interval: 15 minutes Protocol: MQTT/HTTPS Uptime: >99% Retention: 24h minimum real-time | Network failure/recovery testing |
|  | FR-DAT-03 | User Data Access | Mobile app: Real-time dashboard Web interface: Historical graphs Export: CSV download capability API: RESTful endpoints | Cross-platform access testing |
|  | FR-DAT-04 | Growth Analytics | DLI calculation VPD computation Growth rate estimation Yield prediction | Algorithm validation with actual growth |

Non-Functional Requirements

| Category | Requirement ID | Requirement Description | Specification | Verification Method |
| --- | --- | --- | --- | --- |
| Reliability | NFR-REL-01 | System Uptime | Operational uptime: >99% MTBF: >10,000 hours Mean Time To Repair: <4 hours | Continuous operation stress test |
|  | NFR-REL-02 | Sensor Accuracy | Temperature: ±0.5°C Humidity: ±3% RH pH: ±0.1 pH CO₂: ±50 ppm | Calibration against certified references |
|  | NFR-REL-03 | Network Reliability | Local connectivity: 99.5% Cloud sync: 99% successful Reconnection: <60 seconds after outage | Network failure simulation |
|  | NFR-REL-04 | Power Resilience | Load shedding survival: 4 hours Brownout operation: 180-250V Surge protection: 2kV line-to-line | Power quality simulation testing |
| Response | NFR-RSP-01 | Control Response Time | Sensor to actuation: <2 seconds User command execution: <1 second Alert generation: <30 seconds | Real-time performance measurement |
|  | NFR-RSP-02 | User Interface Response | App loading: <3 seconds Data refresh: <5 seconds Graph rendering: <2 seconds | UI/UX performance testing |
|  | NFR-RSP-03 | Cloud Communication | Data upload: <5 seconds Command download: <2 seconds Sync interval: 15 minutes | Network latency testing |
|  | NFR-RSP-04 | Emergency Response | Critical alert delivery: <60 seconds Safety shutdown: <500ms Backup activation: <1 second | Emergency scenario simulation |
| Safety | NFR-SAF-01 | Electrical Safety | NRCS VC 8035 compliance IP44 enclosure rating Earth leakage protection: 30mA Double insulation requirement | Certified laboratory testing |
|  | NFR-SAF-02 | Water-Electricity Separation | SELV circuits near water: <24V DC Waterproof connectors: IP67 rating Drip loops and drainage | Water exposure testing |
|  | NFR-SAF-03 | Thermal Safety | Max surface temperature: <60°C Over-temperature shutdown: 85°C Fire-resistant materials: UL94 V-0 | Thermal imaging and testing |
|  | NFR-SAF-04 | Chemical Safety | Food-grade materials Leachate testing: SAZAS compliant Child-safe reservoir access MSDS documentation provided | Material certification review |
| Maintainability | NFR-MNT-01 | Modular Design | Component replacement: <15 minutes Tool requirements: Basic only Spare part availability: <5 business days | Physical maintenance simulation |
|  | NFR-MNT-02 | Software Updates | OTA update capability Rollback functionality Update duration: <10 minutes No data loss during update | Update process testing |
|  | NFR-MNT-03 | Calibration Simplicity | pH calibration: <10 minutes Sensor calibration: Guided process Calibration reminders automated | User testing with novices |
|  | NFR-MNT-04 | Documentation | Setup guide: <30-minute target Troubleshooting flowchart Multilingual: EN/AF/ZU Video tutorials available | User comprehension testing |
| Power Efficiency | NFR-PWR-01 | Power Consumption | Maximum draw: 150W continuous Average consumption: 80-120W Standby power: <1W Monthly target: <90 kWh | Power meter measurement |
|  | NFR-PWR-02 | Energy Management | Load shedding response Peak demand reduction Solar readiness Battery efficiency: >85% | Power management testing |
|  | NFR-PWR-03 | Component Efficiency | LED efficiency: >2.5 µmol/J Pump efficiency: >40% PSU efficiency: >85% | Component specification verification |
| Usability | NFR-USG-01 | Setup Time | Initial setup: <2 hours Plant changeover: <30 minutes Configuration: <15 minutes | First-time user testing |
|  | NFR-USG-02 | User Interface | Mobile app: 90% task success rate Error recovery: <3 steps Learning curve: <1 day proficiency | Usability testing with target demographic |
|  | NFR-USG-03 | Accessibility | Font size adjustable Color contrast compliant Voice control ready Physical buttons for critical functions | Accessibility standard testing |
| Environmental | NFR-ENV-01 | Operating Conditions | Temperature: 15-35°C ambient Humidity: 30-80% non-condensing Altitude: 0-1500m above sea level | Environmental chamber testing |
|  | NFR-ENV-02 | Noise Level | Operational noise: <40 dB at 1m Sleep mode: <30 dB Vibration: Minimal detectable | Sound level meter testing |
|  | NFR-ENV-03 | Material Sustainability | Recyclable materials: >80% RoHS compliance Local sourcing preference End-of-life plan | Material analysis and sourcing audit |
| Cost | NFR-CST-01 | Unit Cost | Manufacturing cost: <R 4,500 Retail price: R 7,500-R 9,999 Lifetime: 5 years minimum | Bill of Materials analysis |
|  | NFR-CST-02 | Operating Cost | Monthly electricity: <R 350 Monthly consumables: <R 150 Maintenance: <R 500/year | Operational cost modeling |
|  | NFR-CST-03 | ROI Period | Payback period: <24 months Break-even: 200 units Production scale: 1,000 units/year | Financial projection analysis |

Specific Requirements

| Requirement Type | Local Consideration | Specification | Compliance Standard |
| --- | --- | --- | --- |
| Power Quality | Load Shedding Adaptation | 4-hour battery backup minimum Brownout operation: 180-250V Surge protection: 2kV | Eskom grid conditions |
| Connectivity | Network Reliability | Multi-network SIM support Data cost: <R 50/month Offline operation: 7 days | SA network coverage maps |
| Safety Standards | Legal Compliance | NRCS VC 8035 mandatory SANS 10142 wiring compliance Local certification required | South African National Standards |
| Water Usage | Conservation Requirements | Recirculation efficiency: >90% Daily consumption: <5L/kWh Drought mode capability | Municipal water restrictions |
| Local Support | Service Infrastructure | Spare parts: Johannesburg stock Technical support: Local language Warranty: 2 years minimum | Consumer Protection Act |

Verification and Validation Summary

| Phase | Requirement Coverage | Test Method | Success Criteria |
| --- | --- | --- | --- |
| Unit Testing | Individual components | Bench testing | 100% component specification met |
| Integration Testing | System interaction | End-to-end testing | All interfaces functional |
| Environmental Testing | NFR-ENV series | Chamber testing | Operation within specified ranges |
| User Acceptance | Usability requirements | Pilot program (10 users) | 90% satisfaction rating |
| Certification Testing | Safety requirements | NRCS-approved lab | Full compliance certification |
| Field Testing | Reliability requirements | 90-day continuous operation | 99% uptime achieved |