# Zovuyo Innovations — Smart GrowKit

Living Architecture & Team Lead Document

Version 0.1 — Living draft · 7 September 2026 · Audience: mechanical, electronics, firmware, cloud, advisors

This pack leads incoming engineering teams. Structure first, then IoT layers, then control-loop algorithms. Greenhouse / Greenspace farmer systems are phase two — not the first ship.

# 1. Labelled drawings

## 1.1 Three-tier structure

Figure 1 — Three hydroponic beds, shared 20 L reservoir, supply riser, gravity return, control box.

## 1.2 IoT architecture (sensors → controls)

Figure 2 — Five layers: Sensors, Controller (ESP32), Actuators, Server/Cloud, Controls/UX, with closed-loop feedback.

## 1.3 Control loops & safety supervisor

Figure 3 — Lighting, pump, temperature, humidity, pH loops + always-on safety supervisor. Offline-first.

# 2. Product north star

Smart GrowKit: compact IoT three-layer hydroponic appliance for urban SA homes.

Demo defaults: ~0.6×0.4 m footprint, 1.2–1.8 m height for three beds, 20 L reservoir, pump 15 min ON / 45 min OFF, ESP32, HiveMQ→Node-RED→Influx→Grafana+Telegram.

Retail ambition R5k–R8k class. Primary product = home GrowKit; greenhouse later.

# 3. Team map

| Team / lane | Owns | Primary artefacts |
| --- | --- | --- |
| A — Structure & hydro | Frame, beds, plumbing, reservoir, LED mounts | CAD, mechanical BOM, § structure |
| B — Power & electronics | PSU, fuse, bucks, relays, control box | Schematics, proto/PCB |
| C — Firmware | ESP32 loops, drivers, safe mode, MQTT | PlatformIO repo |
| D — Cloud & UX | Broker, Node-RED, Influx/Grafana, Telegram | Dashboards, topic contracts |
| E — Product / compliance | Brand, POPIA, SANS/NRCS roadmap, demo script | Decision log, risks |

# 4. Three-layer structure summary

| Tier | Role | Typical crop | Notes |
| --- | --- | --- | --- |
| Tier 3 (top) | Max ventilation | Herbs | Warmest / most airflow |
| Tier 2 (middle) | Main production | Lettuce NFT | Primary demo bed |
| Tier 1 (bottom) | Nursery | Seedlings / microgreens | Protected |

MVP plumbing: one pump feeds up; gravity return down; shared reservoir sensing (pH/level/temp) — not per-bed probes yet.

# 5. IoT layers (names to use everywhere)

1 Sensors — DHT, DS18B20, pH, level (± EC/CO₂ later)

2 Controller — ESP32, PlatformIO, SD log, Wi-Fi

3 Actuators — pump relay, LED PWM, fan PWM (± heater/dose later)

4 Server/cloud — HiveMQ, Node-RED, InfluxDB, Grafana, Telegram

5 Controls/UX — profiles, setpoints, overrides, vacation, maintenance

Hard rule: layers 1–3 keep plants alive offline; cloud is observability + remote config.

# 6. Control-loop algorithms

Common cycle: SENSE → DECIDE → ACT → LOG → PUBLISH MQTT

L1 Lighting — 16/8 photoperiod, soft PWM ramp

L2 Pump — 15 ON / 45 OFF; inhibit on low level

L3 Temperature — fan cool / heater heat with hysteresis

L4 Humidity — fan PWM / VPD assist

L5 pH — sample every 15 min; alert only in MVP (auto-dose later)

Safety supervisor — low water, over-temp, sensor loss, Wi-Fi loss → SAFE MODE

# 7. Decision log (seed)

2026-09-07 — Primary product = home Smart GrowKit; greenhouse later (Oluhle)

2026-09-07 — Demo defaults: 20 L, 15/45 pump, ESP32, free MQTT stack

2026-09-07 — Physical intent = three stacked beds with shared nutrient delivery (Oluhle)

# 8. Open decisions

D1 Exact prototype height (1.2 m vs 1.5–1.8 m)

D2 Primary DC rail (12 V vs 24 V front-end)

D3 Bed tech per tier (all NFT vs mixed)

D4 Brand mark on demo UI (Zovuyo / Smart GrowKit)

D5 EC & CO₂ on first wet demo?

Full prose, mermaid, MQTT contract, and checklists: see companion Markdown in this pack.